Course Handbook & Walkthrough

Maschine MK2 Fundamentals

February 2017

Liam Chaytor

Acateor@live.com

www.Acateor.weebly.com

© 2017 ACATEOR PRODUCTIONS ALL RIGHTS RESERVED

Course Overview

Course Overview and Contents . . . . . 2

- Introduction . . . . . 3

- Course Description and Aims . . . . . 3

Hardware Overview . . . . . 4

- Maschine Overview . . . . . 4

- Master Volume, Swing and Tempo . . . . . 5

- Pads, Groups and Transport . . . . . 5

Navigation and Functions . . . . . 6

- Navigating Control Mode . . . . . 6

- Pad and Transport Functions . . . . .6 - 7

Browsing and Loading the Library . . . . . 8

- Browsing and Loading Sounds/Sample and Instruments . . . . . .8

- Browsing and Loading Effects . . . . . 8 - 9

Recording and Looping . . . . . 10

- Setting the tempo . . . . . 10

- Recording a Pattern . . . . . 10

- Pitching Sounds/Samples for recording . . . . . 10 - 11

- Quantize and Grid . . . . . 11

- Recording Automation . . . . . 11

Note Repeat and Arpeggiator . . . . . 12

- Note Repeat . . . . . 12

- Pitching Note Repeat . . . . . 12

- Arpeggiator . . . . . 12

- Manual nd Chord Arpeggiator . . . . . 13

Pad Mode and Keyboard Mode . . . . . 14

- Setting up Keyboard Mode . . . . . 14

- Setting up the Scale and Chord . . . . . 14

Patterns and Scenes . . . . . 15

- Patterns . . . . . 15

- Recording Patterns . . . . . 15

- Scenes . . . . . 15

- Arranging with Scenes . . . . .16

- Creating and Navigating Scene Pages . . . . . 16

Shortcuts and Workflow . . . . . 17

- Master Workflow . . . . . 17

- Pad Workflow . . . . . 17

- Select, Navigate and Fixed Velocity . . . . . 17

- Mixing Mode . . . . . 17

- Shortcut Table . . . . . 18

- Terminology . . . . . 18

Introduction

Course Description

This course is designed to break a new user into the Native Instruments software and Maschine Hardware. The key points are covering Navigation, Recording/Playback, Loading Sounds and Plug-ins, Understanding Patterns and Scenes, Understanding Keyboard Mode and using shortcuts to enhance workflow.

Course Aims

When learning any new DAW or Instrument, it is important to remember that it is not easy at first. However, through using this guide, the skills you will gain include:

- Understanding of Native Instruments Software

- Navigate effectively using the Maschine Hardware

- Understand the key functions of Maschine

- Develop an understanding of Patterns and Scenes

- Develop a smooth workflow on the Maschine Hardwar

Maschine Hardware Overview

The Maschine is split into 4 main categories, Master, Pads, Groups and Transport.  ‘Browse’ is located at the top left of the hardware, alongside the screens.

The most top left button in the corner is ‘CONTROL’.

‘CONTROL MODE’ is the home page of Maschine.

Below is a labeled diagram of the Maschine MK2, pointing out the main sections and buttons of the hardware.

White button strip

Control

Browse

Screens

Control Knobs

Pad Section

Master

Section

Master Wheel

1

2

3

4

5

6

7

8

Transport Bar

Groups

© 2017 ACATEOR PRODUCTIONS ALL RIGHTS RESERVED

Master

The master controls are self-explanatory, controlled by selecting the function and then turning the Master Wheel.

Volume is the master volume of the software, and is controlled with the master wheel.

Swing refers to how the track plays back. If the track is at 0% swing (deactivated) then it is played back ‘Straight’. If the track is at 100% Swing, the playback will have a curved motion (swinging motion, back and forth) to it.

Tempo controls the master tempo, which is the speed of the track. The tempo is measured in Beats per Minute, or, BPM for short.

To more finely make adjustments, hold ‘SHIFT’.

Note Repeat and Tap will be covered later on in this course. (Page 12)

Pads

The Pads are a 4 x 4 midi pad surface. ‘Sounds/Samples’ can be loaded into each individual pad or ‘Groups’ can be loaded (all 16 sounds loaded directly).

Along the side of the pads is a column of important functions, which are always used in any creative process on Maschine.

Groups

A ‘Group’ holds 16 sounds, which can be played back live from the pads. To switch between groups, simply click it, and 16 new sounds are available.

To create a new group, select the ‘white-lit’ group button (white-lit represents empty).

To switch groups, simply select the group you would like to switch to, by clicking the button.

Transport

The transport bar is where playback, looping and recording are enabled. The ‘SHIFT’ button, functions identically to the ‘Shift’ key of a PC or Laptop Keyboard.

Navigation and Functions

Navigating Control Mode

In ‘Control Mode’, use the white buttons (left screen) to switch between ‘Master, Group or Sound’.

The right screen shows either the ‘CHANNEL’ or ‘PLUG-IN’ (switched using white buttons).

To switch the page view, use the arrows underneath the ’BROWSE’ button to navigate back and forth.

In ‘CHANNEL’ use the white buttons to navigate between ‘INPUT, OUTPUT, GROOVE and MACRO’.

In ‘PLUG-IN’ use the white buttons to navigate between different effects and plug-ins. To navigate the plug-in pages, use the arrows under ‘Browse’.

Pads Functions

Below is a table of some of the buttons and their combinations. These have been filtered down to the most important to know. These functions are mostly used for editing midi information and navigating to the arrangement features of Maschine.

| SCENE | Enter Scene Menu |
| --- | --- |
| PATTERN | Enter Pattern Menu |
| PAD MODE | Enter Pad Mode |
| DUPLICATE | Copy and Paste |
| SOLO | Solo a group or sound |
| MUTE | Mute a group or sound |
| SHIFT + PAD MODE | Enter Keyboard Mode |
| SHIFT + PAD 1 | Undo |
| SHIFT + PAD 2 | Redo |
| SHIFT + PAD 5 | Quantize |
| SHIFT + PAD 6 | Quantize 50% |
| SHIFT + MUTE | Choke |

Transport Functions

The transport holds what you would expect to see in any other DAW. Playback start/stop, record enable and Loop on/off. These shortcuts and buttons will be used no matter the project.

| PLAY | Starts Playback of the Track |
| --- | --- |
| REC | Enables Recording |
| ERASE | Deletes Patterns and Scenes |
| SHIFT + ERASE | Deletes Groups and Sounds |
| GRID | Changes the midi grid note value |
| RESTART | Plays the track from the beginning of the loop or track |
| SHIFT + RESTART | Loop on/off |
| SHIFT + PLAY | Metronome on/off |
| SHIFT + REC | Count in (4 Beats before recording and playback starts |

Browsing and Loading the Library

Browsing and Loading Sounds/Samples and Instruments

Select the pad you want to assign a ‘SOUND/SAMPLE’ to by tapping it.

Select ‘BROWSE’, use the white navigations buttons above the screen to find ‘SOUND, SAMPLES or INSTRUMENTS’.  (Right Screen)

Use the control knob to select ‘PRODUCT’ or ‘TYPE’.

Once you have chosen a ‘sound/sample or instrument’, press the white button above ‘LOAD’ to assign it to the pad. (Right Screen)

Samples have a ‘PRE-HEAR’ option, assigned to ‘CONTROL KNOB 4’ on the left screen. Pre-hear allows you to hear a snippet of the sample as you browse.

Browsing and Loading Groups

Select the group you want to assign a ‘Kit’ to by pressing it.

Select ‘BROWSE’, use the white navigation buttons above the screen to find ‘GROUPS or SAMPLES’.  (Right Screen)

Use the control knob to select ‘Product or Type’.

Once you have chosen a ‘KIT’, press ‘LOAD’. (Right Screen)

To auditions kits, press ‘PLAY’ to start playback of pre-recorded patterns, press ‘PATTERNS’ to see/hear the presets, switch between them by selecting the pads (if a pad is not coloured, it means it is empty).

If you don’t like the patterns, hold ‘SHIFT + ERASE’ and select the pattern you wish to delete by selecting the corresponding pad.

Browsing and Loading Effects

Select the ‘SOUND, GROUP or MASTER’ track while in ‘Control Mode’ by using the white navigation buttons (Left Screen).

Press ‘SHIFT + BROWSE’, to open the effects library.

Use the control knob (left) to select a vendor, and the other control knob (right) to select the effect. Press the white button above ‘LOAD’ to assign the effect to the track.

Vendor – The company or artist that creates the Sounds/Samples, Groups or Effects.

To delete an effect, enter ‘PLUG-IN’, press ‘SHIFT’ and select ‘REMOVE’ on the left screen. To mute an effect, ‘PLUG – IN’, press ‘SHIFT’ and click ‘BYPASS’.

Recording and Looping

When recording live looping, it is best to leave the ‘LOOP’ feature on, by pressing ‘SHIFT + RESTART’. The ‘RESTART’ light being on/off represents the on/off toggle.

Setting the tempo

To set the tempo of your track, use the ‘TAP TEMPO’, or, the manual-master ‘TEMPO’ feature.

To set tempo with Tap Tempo, hold ‘SHIFT’ and tap the ‘NOTE REPEAT’ button at the speed you want to set the BPM as.

To set manually, select master ‘TEMPO’ and use the master wheel to select. To more finely edit the tempo, hold ‘SHIFT’.

Recording a Pattern

Loops are referred to as patterns.

More information about patterns is further on in the course.

To start recording, select ‘REC’. It will show red when recording is enabled.

To enable recording with a 4 beat count in, press ‘SHIFT + REC’.

Use the drum pads record a pattern.

If you make a mistake and wish to delete all of the midi information within the pattern, press ‘SHIFT + PAD 9’ to clear all.

To delete the midi information from one sound, hold ‘Erase’ and hold the Pad at the time you want to delete the information.

Pitching sounds/samples for recording

If you have a sample or sound, for example an ‘808 snare drum’, it is possible to quickly pitch it up or down in various ways.

The first way to pitch a sound/sample is by selecting the pad, by tapping it, and entering ‘KEYBOARD MODE’.

Keyboard mode pitches the sound across the chromatic scale (all keys of a keyboard) till it is in audible by octave.

The second way is to ‘tune’ the sound.

To do this enter ‘CONTROL MODE’ (top right), select ‘SOUND’ (left screen) and enter page 2 (using navigation buttons under ‘BROWSE’).

Page 2 of the Sampler plug-in is titled ‘PITCH / GATE’.

To adjust the ‘TUNE’ use ‘CONTROL KNOB 1’, to more finely adjust, hold ‘SHIFT’ while turning.

Quantize

To quantize the pattern press ‘SHIFT + PAD 5’.

To quantize the pattern more loosely, use ‘QUANTIZE 50%’ by selecting ‘SHIFT + PAD 6’.

To quantize a single pad, press ‘SHIFT + SELECT’ to enter the ‘EVENTS’ display.

Select the pad, then, use an above quantize function to complete the action.

Automatic Quantization is where anything you record using the drum pads are then automatically quantized as it is played back. To enable this feature, hold ‘SHIFT + GRID’ and set ‘QUANTZIE MODE’ to ‘Record’.

*Quantize – moves the midi information to the closest value within the midi grid.

Grid

The grid sets, which note value, you want the quantization to apply.

Hold ‘GRID’ and select ‘GRID’ (located top left) and ‘STEP’ (left screen), opening the Grid Step Display. Using the right screen as reference, select the grid you want (1/8 and 1/16 being most common) by selecting the drum pad corresponding to it.

Recording Automation

To record automation, load a plug-in to a group, which has a recorded pattern.

Hold ‘AUTO’ (located to the left of ‘CONTROL KNOB 1’) while playback is on and turn the ‘CONTROL KNOBS’ changing the parameters of an effect to record the automation.

Automation effects all of the patterns within a group, and is applied at all times to that group.  Meaning the automation will loop as long there is playback from that group, regardless of the pattern.

To get around this, jam the cut-off to 100% to normalize the effect after you’ve recorded your automation (meaning no effect going through the mix). This could vary depending on what you are working on.

Note Repeat and Arpeggiator

Note repeat, simply, repeats a note consecutively in time to the master tempo, at a set rate. This rate can be changed for a live effect. The Arpeggiator works similarly, but instead, playing a range of selected pads that are assigned to notes.

Note Repeat

Select ‘Note Repeat’ in the Master section of the hardware (right of the master wheel) to enter it. To keep the display showing without holding the button, select ‘NOTE RPT’ on the left screen.

Across the right screen are values ‘1/8, 1/16, 1/16 T and 1/32’. This represents the speed of the Note Repeat. These values can be changed by selecting them and changing the ‘RHYTHM RATE’ by turning ‘CONTROL KNOB 3’. The ‘RHYTHM UNIT’ is type of note, ‘Normal, Triplet and Dotted’.

Load a ‘Snare drum sample’ and enter ‘NOTE REPEAT’. Hold the pad down and it will playback at the rate set (right screen).

While holding down the pad, switch between the 4 values randomly to create build-up rolls and fills.

Pitching Note Repeat

To pitch note repeat, use the previous ‘Keyboard Mode’ pitching technique.

Load ‘KEYBOARD MODE’ of the snare drum sample. Set a scale if you wish, or use the chromatic scale.

Set the note repeat value to ‘1/16’ (right screen) and play the pitched pads randomly to create a pitched snare drum roll.

Whilst playing different pitches, try switching between the different Note Repeat values like done previously.

Arpeggiator

Arpeggiators are used for melodic instruments, such as synth leads and guitars.

The set up process of ‘Note Repeat’ are the same, and the ‘note repeat values’ can still be switched live.

To use the Arpeggiator there are 2 options. The first manually playing the chords yourself, and the second using the chord feature in keyboard mode.

Manual Arpeggiator

Load a melodic instrument from the Instrument library by selecting ‘BROWSE’. Navigate to ‘INSTRUMENTS’ (left screen), using the control knobs and ‘LOAD’ an instrument.

Enter ‘Keyboard Mode’ and set the scale type to ‘Minor’ and enable ‘FIXED VEL’.

Enter ‘Note Repeat’ and set the value to ‘1/16’.

Hold multiple drum pads and the Arpeggiator will play the corresponding notes.

Chord Arpeggiator

Using chord Arpeggiator, only one pad needs to be held whilst the notes play.

To do this, re-enter ‘KEYBOARD MODE’ and set the ‘CHORD MODE’ to ‘HARM’ and the ‘CHORD TYPE’ to ‘1 – 4 – 7’ and ensure ‘SCALE TYPE’ is set to ‘MINOR’.

Enter ‘Note Repeat’ and hold a pad. The notes of the chord will playback.

Try switching between note repeat values (left screen) as done previously.

Keyboard mode

Keyboard mode takes a sound and spreads it across the chromatic scale, (white and black keys of a piano) until the sound can’t be processed any further or becomes inaudible.

Setting up Keyboard Mode

Press ‘SHIFT + PAD MODE’

OR

Enter ‘Pad Mode’ and select ‘KEYBOARD’ with the white buttons (above the left screen).

To make the display show PAD/KEYBOARD MODE without holding the button, select ‘PAD/KEYBOARD MODE’ on the top left corner, of the left screen.

This function applies to: Pad mode, Navigate, Pattern and Scene.

If you want to have the pads play a consistent volume, select ‘Fixed Vel’. This stands for Fixed Velocity, which sets the volume of the midi information to a set level.

To set this level, go to Page 2 (using the arrows) and use ‘CONTROL KNOB 1’ to set the level.

Setting up the Scale and Chord

On the left screen we have the ‘SCALE’ page, which allows you to set a ‘TYPE’ of scale (Major or Minor) and activate ‘CHORD MODE’.  Chord mode sets a single pad (only while in ‘keyboard mode’) to play the full chord by triggering the other notes automatically.

On the right screen, the 4 x 4 diagram represents the drum pads. The diagram contains the note information of the current scale. As the type of scale changes the note display change accordingly.

To change the root note of the scale, use the ‘Semi- and Semi+’ (white buttons at the top right). To change the octave, use the ‘OCT- and OCT+’.

Now we will set up the ‘SCALE TYPE and CHORD MODE’:

Scale: A Minor

Chord: 1-3-5-7 (7th chords)

- Enter ‘KEYBOARD MODE’

- Set ‘SCALE TYPE’ to Minor

- Set ‘CHORD MODE’ to Harm (Harmonic)

- Set the ‘CHORD TYPE’ to ‘1-3-5-7’ (7th chord)

- Set the root note to ‘A’ and then set the octave to ‘1’

Patterns and Scenes

Patterns

Patterns are loops of midi information and are used to construct a piece of music. A pattern is set to a number of bars and loops till changed or turned off.

Patterns are bound to their group, meaning if you record a pattern on Pad 1 in Group A, that pattern becomes A1.

Enter the Pattern View by selecting ‘PATTERN’ in the column of buttons in the Pads section.

The patterns are displayed in the top right hand corner, similarly to the Keyboard mode display.

Recording Patterns

To create a new pattern, enter ‘PATTERN MODE select ‘Pad 1’ and set the ‘PATTERN LENGTH’.

To record the pattern, exit ‘PATTERN MODE’ (by reselecting pattern or selecting control mode) press ‘SHIFT + REC’ and record a pattern.

Repeat this process to record two patterns (4 bars length each) to use in the Scenes example.

Scenes

Scenes are what are used when arranging your track.  A scene holds one pattern from each group, and like patterns, can be set for a specific amount of bars.

Scenes are laid out across the timeline to create the structure to music.

When playing the track back, by turning ‘Loop’ off you can hear the track across the timeline. However, if ‘Loop’ is on, by using the drum pads you can switch between scenes, and create the tracks arrangement as you go, or create a remix type effect.

Scenes can have a set ‘LENGTH’, set-up identically to the patterns, by entering ‘SCENE’ and using ‘CONTROL KNOB 4’ to adjust.

If the length is left at ‘AUTO’ and ‘LOOP’ is disabled, the scene will change when the longest pattern has resolved.

If there are two patterns in a scene, Pattern 1 is 2 bars long, and Pattern 2 is 8 bars long, Pattern 1 will repeat 4 times, until Pattern 2 has resolved.

In this Example, the instrument groups have patterns recorded, ready for playback.

The next step is to order the patterns using Scenes.

Arranging with Scenes

There is a simple process to arranging with scenes.

Enter scene mode by selecting ‘SCENE’ in the pads section.

Create a new scene by selecting pad 1, the same way as creating a pattern.

Set the length, of the scene (or leave at auto if you prefer looped playback).

Click ‘PATTERNS’ and select the pattern of each group, and it will assign to that scene, like a snapshot.

Enter scene mode again, select ‘Pad 2’ to create Scene 2 and repeat the selection process to create your arrangement.

Example.

|  | Scene 1 | Scene 2 |
| --- | --- | --- |
| Group A | Pattern 1 | Pattern 2 |
| Group B | Pattern 1 | Pattern 2 |
| Group C | N/a | Pattern 1 |

Creating and Navigating Scene Pages

To create a new page of Scenes, use the white buttons above the right screen marked ‘< and +’ and select ‘+’ to create a new page.

After, the button changes to ‘>’ to allow navigation between page.

Shortcuts and Workflow

Master Workflow

Master Wheel – Adjusts Tempo without selecting the master ‘TEMPO’ key (If no other functions are selected).

To change how finely the wheel adjusts the master parameters, hold ‘SHIFT’ while turning.

To adjust the volume of a pad or group quickly, Select ‘VOLUME’, hold the pad or group, and turn the master wheel.

The same process can be used to adjust the ‘SWING’ feature.

Pads Workflow

‘Mute/Solo’ + ‘Pad/Group’ = Mute Selection

To un-mute or mute everything, hold ‘MUTE’ and select ‘NONE’ OR ‘ALL’ on the right screen.

‘SHIFT’ + ‘MUTE’ = CHOKE

(Used for cutting long tails off of a sample during/after playback)

Select

To change instrument without exiting keyboard mode, press ‘SELECT’, tap ‘Pad 2’ and exit view by pressing ‘SELECT’.

To change multiple parameters, hold ‘SELECT’ and press ‘MULTI’ (right screen).

Navigate

Navigate controls how the software looks. Press ‘NAVIGATE’. Select and de-select ‘BROWSER, MOD and MIXER’, and watch the software respond.

Fixed Velocity

To set ‘FIXED VELOCITY’, enter ‘PAD MODE’ and select ‘FIXED VELOCITY’ (Left screen).

To set the velocity meter, scroll to Page 2 using the navigation arrow (under browse) and use ‘CONTROL KNOB 1’ to set it.

Mixing Mode

To enter mixing mode select ‘SHIFT + SAMPLING’. ‘GROUP and SOUND’ are available to be mixed in the editor, using the white buttons above to navigate between them.

When mixing in ‘SOUND’ mode, use the navigation arrows (under browse) to switch from sounds 1-8, to 9-16.

Using the right screen, navigate back and forth between ‘LEVEL’ and ‘PAN’. Adjusting all parameters with the control knobs.

Shortcut Table

| Play | Starts Playback of the Track |
| --- | --- |
| Rec | Enables Recording |
| Erase | Deletes Patterns and Scenes |
| SHIFT + Erase | Deletes Groups and Sounds |
| Grid | Changes the midi grid note value |
| Restart | Plays the track from the beginning of the loop or track |
| SHIFT + Restart | Loop on/off |
| SHIF + Play | Metronome on/off |
| SHIFT + Rec | Count in (4 Beats before recording and playback starts |

| Scene | Enter Scene Menu |
| --- | --- |
| Pattern | Enter Pattern Menu |
| Pad Mode | Enter Pad Mode |
| Duplicate | Copy and Paste |
| Solo | Solo a group or sound |
| Mute | Mute a group or sound |
| SHIFT + Pad Mode | Enter Keyboard Mode |
| SHIFT + Pad 1 | Undo |
| SHIFT + Pad 2 | Redo |
| SHIFT + Pad 5 | Quantize |
| SHIFT + Pad 6 | Quantize 50% |
| SHIFT + Mute | Choke |

Terminology

Workflow, Pattern, Scene, Quantize, Mix, Tempo, Swing, Fixed Velocity, Arranging

END OF COURSE